Explain how the thermostat supports the peripherals used in the project.

The I2C peripheral is used to read the temperature sensor. Within the code, an I2C instance is created, the temperature and humidity sensor is initialized, and the temperature is accessed through the sensor in the getFahrenheit function.

The GPIO peripherals support all of the visible functionality of the code. The buttons toggle the system state between off, heat, and cool and increment and decrement the set temperature point. The lights display the state by fading in and out, being solid, or being off depending on the temperature set point. The display displays the datetime, state, current temperature, and set temperature to the user. Each component has assigned values from the GPIO board. The red light is GPIO 18, the blue light is GPIO 23, and the LCD display uses many GPIO values (D 17, 27, 5, 6, 13, and 26). Those are reflected in physical connections on the board. The green button uses GPIO 24, the red uses 25, and the blue uses 12. All of these components have their own classes and class methods (ex, when_pressed) that are used after the connection is established.

The UART peripheral supports the code’s client-server architecture. The serial port “ser” is set up, and state information is written to the server. A similar logic could be used to communicate with the cloud, which Raspberry Pis, microchips, and freeform all support.

Explain how the thermostat connects to the cloud via Wi-Fi.

The thermostat could use a client-server setup to connect to the cloud via wifi, and already has that logic in place. The server would have to be replaced with a cloud server. There is an example of an IoT client-server setup with Raspberry pi in the works cited by kgremban. Microchips/microcontrollers are also perfectly capable of connecting to the cloud- an example is in the references from Lofstad, J of a clock app with cloud connection. Freescale is also capable of using Python to collect to the cloud via wifi, and there is an example by Snell, D in the references.

Discuss the architectures’ capabilities that support the code.

This code was designed for a Raspberry Pi, so it does support Pis.

Microcontrollers are a type of microchip (PCB HERO, 2024), and microcontrollers can be programmed using CircuitPython, which is an adaptation of Python designed specifically for microcontrollers (Adafruit Learning System, 2026). The code would need to be adapted to CircuitPython, but all the logic would be the same. There are many different types of microchips (ASML, 2022). Not every microchip would have these capabilities- memory chips, for example, would not be capable of performing these tasks as they are made to store memory, not run code (ASML, 2022). A microcontroller, specifically, would need to be used if going with the CircuitPython route. It would also be needed if running code with just Python.

Freescale semiconductors can also run Python code.

References

ASML. (2022). The Basics of Microchips. Www.asml.com. https://www.asml.com/en/technology/all-about-microchips/microchip-basics

Johan Lofstad. (2020, March 9). Cloud Computing in IoT. Microchip.com. https://www.microchip.com/en-us/about/media-center/blog/2020/cloud-computing-in-iot

kgremban. (2024, July 10). Connect Raspberry Pi to Azure IoT Hub - Azure IoT Hub. Microsoft.com. https://learn.microsoft.com/en-us/azure/iot-hub/raspberry-pi-get-started

Lofstad, J. (2020). Home Automation Home Automation -Weather Clock. Retrieved March 1, 2026, from https://ww1.microchip.com/downloads/en/DeviceDoc/Home-Automation-Weather-Clock-DS50002962A.pdf

Snell, D. (2014, May 22). Cloud-Connected ZigBee Thermostat Demo [Review of Cloud-Connected ZigBee Thermostat Demo]. NXP Community; NXP Semiconductors. https://community.nxp.com/t5/NXP-Designs-Knowledge-Base/Cloud-Connected-ZigBee-Thermostat-Demo/ta-p/1120673

Welcome to CircuitPython! (2026). Adafruit Learning System. https://learn.adafruit.com/welcome-to-circuitpython/what-is-circuitpython

What is the difference between microcontroller and chip？. (2024, October 28). PCB HERO. https://www.pcb-hero.com/blogs/lickys-column/what-is-the-difference-between-microcontroller-and-chip